# DCS-2120 Firmware Release Note

Firmware: DCS-2120-DLNK-0103c

Hardware: A2G

Date: 15th Aug, 2007

Bugs fixed:

The UPnP port forwarding can not work

Firmware: DCS-2120-DLNK-0103b

Hardware: A2G

Date: 20th July, 2007

1. New features:

* D-Link NTP server

* D-Link DDNS (WorldWide, CN)

* Mute function

* Vista support

2. Changed features:

* Add audio/video extension support

Firmware: IP7137-DLNK-0200f (1.00)

Hardware: A1

Date: 24th Mar, 2006

Problems Resolved:
1. DUS test report: Camera's web interface can be seen from computer but network page can't display and the pink LED shows up.

2. DCAN test report: no way to enter a key in WEP 128bit ASCII mode even it's valid key. And always display “Key must have exactly 13 characters” whatever key is entered.

3. DUS test report: If the HTTP port is not on 80, Reboot or Set back to default will add the additional port after the port number, linking it to the wrong page.

4. DCAN test report: in image settig page, if you select +5 and then hit “preview” the settings are committed.  If you then go to another tab and back the previous +5 will show as 0 yet the settings will still be committed.

Enhancements:
   N/A

Firmware: IP7137-DLNK-0201c+ (1.01)

Hardware: A2G

Checksum: 00807538.

Date: 4 Sept., 2006

Problems Resolved:
1. Add RTSP authentication function

2. Allow user to configure RTP/RTCP port

3. Fix issue in D-view cam s/w. (error message pops up when log in to DCS-2120 MD web page)

4.Not to show Vivotek F/W rev. number when upgrade F/W on the webpage.

Enhancements:
   N/A

Firmware: DCS-2120-DLNK v1.02 (0202a)

Hardware: A2G

Checksum:00803450

Date: 16 Jan., 2007

Problems Resolved:

- Server can not connect to AP(DI-624) when set WAP-PSK key to "star*()ring".

- Time on the uploaded images on FTP is 8 hours ahead of the Camera time.

Enhancements:
   N/A