# DCS-900 Firmware Release Note

Firmware: 3.10

Hardware: Bx

Date: Apr 6, 2006

Problems Resolved:

When the video resolution is set to 160x120, frame rate is set to 30, and enable FTP upload, the camera will restart accidental.

Enhancements:

None

=============================================================

Firmware: 3.00 (not backward compatible to ver.Ax)

Hardware: B1

Date: Jan 5, 2006

Problems Resolved:

None

Enhancements:

System:

- Support UPnP

- 10/100 Ethernet port with Auto-MDIX

Video:

- Support 160x120, 320x240, and 640x480 resolution

- Frame rate upgrade to 30fps@QCIF, 25fps@CIF, 10fps@VGA

- Added upside down and mirror function

Management:

- Camera status display included System, Video, Network, and Active Users status

Details please refer to the comparison table below.

| Hardware | DCS-900.Ax | DCS-900.B1 |
| --- | --- | --- |
| Image compression | M- JPEG | M- JPEG |
| CMOS Sensor | Color CMOS Sensor | Color CMOS Sensor |
| Lens | F1.8 f6.0mm Board Lens | F1.8 f6.0mm Board Lens |
| CPU | RDC R1610C | DL7200 |
| RAM | 512K Byte | 8M Byte |
| Flash ROM | 512K Byte | 2M Byte |
| Minimum Illumination | 2.5Lux | 2.5Lux |
| Power | DC5V, 2.5A | DC 5V 2.5A, |
| Power consumption | 4.25 Watt | 5 Watt |
| General |  |  |
| Max. Users | 10 simultaneous | 10 simultaneous |
| Operating System | RTOS | RTOS |
| Management | --- | Device status, Ethernet status, |
|  |  | Video status, Wireless status, Network status, Active users |
| Tools | Reset, Factory Reset | Test FTP server, Test Email Account, Backup/Restore device configuration, Reset, Factory Reset, Firmware upgrade |
| Power LED | Green | Red |
| Link LED | Amber | Green |
| Video |  |  |
| Image Frame Rate | 20fps@CIF, 6fps@VGA | QCIF@30fps,25fps@CIF, 10fps@VGA |
| Image Size | 320*240/640*480 | 160*120/320*240/640*480 |
| Image Settings | Brightness, contrast, Saturation | Brightness, contrast, Saturation |
| Network |  |  |
| DDNS Support | Yes | Yes |
| UPnP | No | Yes |
| Communication | HTTP, FTP, TCP/IP, UDP, ARP, ICMP, BOOTP, RARP, SMTP, DHCP, PPPoE | HTTP, FTP, TCP/IP, UDP, ARP, ICMP, BOOTP,DHCP, PPPoE, SMTP, DDNS, UPnP |
| protocol |  |  |
| Lan Port | RJ-45, 10/100M auto-sensed | RJ-45, 10/100M auto-sensed, AUTO-MDIX |

=============================================================

| Model Name | Version | Upgrade information | Date |
| --- | --- | --- | --- |
| DCS-900 | 2.10 | Original | 03/10/25 |
| DCS-900 | 2.20 | Add DDNS feature | 03/10/25 |
| DCS-900 | 2.28 | 1. Text of Restart and Reset to Factory default, also, it's description. 2. To keep last setting of Date and Time. | 04/07/13 |