# DES-3010F/3010FL/3010G/3018/3026 & DHS-3010F/3010G/3018/3026

# Firmware Release Note

Firmware: 4.00.018 (PROM：1.01.007)

Hardware: A1/A2/A3G for DES-3000 series, D1/D2/D3G for DHS-3000 series

Date: June 27, 2007

Modification & add new features:

1. Add 802.1X Guest VLAN feature.

2. Allow enable/disable IP 0.0.0.0 checking on IP-MAC-Port binding ARP mode.

3. Add tagged port information on “Show VLAN” command.

4. Ability to set the row number of Terminal Line.

5. Remove the limitation when browse MAC table in Web UI, user must key in MAC address with “-“, for example “12-23-34-45-56-67”

6. Remove all trusted host by one command in CLI, Web, SNMP.

7. Add Loopback detection v2 (STP independent + port based shutdown)

8. Add an OID to show port utilization

9. Add an OID to clear FDB and ARP table (can’t clear FDB entry based on port or VLAN)

10. Extend trusted host number to 10 entries.

11. Add subnet mask parameter on trusted host, ability to input not only one IP for trusted host

12. Auto complete the commands with extra parameter “force_agree”. Instead of poping up (Y/N) message, it will be executed automatically.

13. Add ping command to user privilege

14. Send trap in SNMP and add warning message in SYSLOG while access violation happens for IP-MAC-Port binding and port security

15. Add Password Recovery feature

16. Port description extended from 32 bytes to 128 bytes

17. Remove first product introductory page in Web-UI for security concern.

18. Add Management IP interface with 128bytes description

19. Rename “DLF” to “Unicast” in storm control feature

20. Enhancement on broadcast storm control feature (port shutdown)

Problems Resolved:

- After configuring 802.1x Guest VLAN and restart the switch, the device keeps rebooting.

- While Uploading default configuration file from switch and then download the same configuration from tftp server, some parameters are missing.

- Extra IE windows shown up and wrong model display while configuring SIM members.

- In 802.X MAC-based Access Control implementation with RADIUS server, even there’s only authentication on one port, but there are multi-ports’ auth statistics shown on web.

- Wrong port number display in IP-MAC-Port Binding log message.

Firmware: 3.00.053 (PROM：1.01.007)

Hardware: A1/A2/A3G for DES-3000 series, D1/D2/D3G for DHS-3000 series

Date: March. 30 2007

Modification:

1. Expanding port description from 32 bytes to 128 bytes.

2. Add management IP interface description (128 bytes).

3. Revise maximum trunk group member ports from 8 to 4

4. Remove Web-UI first product introductory page for security concern.

Problems Resolved:

1. Can’t display H/W version correctly in DES-3000 (A3) and DHS-3000 (D3), this firmware must work with the latest PROM code v1.01.007 in order to resolve this problem.

2. Switch sends cold-start trap while executing warm-start

3. TFTP upload configuration fail after switch operates a period of time, because of "allocate memory failure"

4. Slow display via CLI and Telnet when executing "show fdb" command if clipaging function is disabled.

5. SNMPv1, v2 community group are auto created while adding SNMPv3 community group.

6. Can’t enable SMTP via Web-UI.

7. For DES/DHS-3010G, if port 9 is disabled, the port 10 is also disabled.

8. Input “Port description” will cause the port can not forward traffic after switch restart.

9. While monitor MAC address table, the VLAN name column can’t enter more than 10 characters.

10. The CPU Interface Filtering feature on Web-UI for port list setting can not match with CLI.

Firmware: 3.00.036 (PROM：1.01.006)

Hardware: A1/A2 for DES-3000 series, D1/D2 for DHS-3000 series

Date: Sep. 27, 2006

Problems Resolved:

1. Fix dst (day light saving time) command not meet the CLI definition.

2. DES-3018 FW upgrade from 2.00B20 to 3.00B32 will lost configuration.

Firmware: 3.00.032 (PROM：1.01.006)

Hardware: A1/A2 for DES-3000 series, D1/D2 for DHS-3000 series

Date: July. 10, 2006

Problems Resolved:

1.DES-3018 .BIN file write to DES-3010F, port link fail

2. PROM code upgrade to v1.01.006 for new BCM532x series P12 IC introduce, its hardware version is A2/D2.

3. PI-309 DES-3000 series IP-MAC issue.

Firmware: 3.00.025 (PROM：1.01.005)

Hardware: A1 for DES-3000 series, D1 for DHS-3000 series

Date: Jun. 5, 2006

Add new features:

1. MAC notification

2. Cos based on MAC DA/TOS/DSCP

3. IP/MAC Binding

4. BPDU filtering per port

5. Multicast filtering mode

6. Add save log command.

7. Support new module: DEM-201FL

8. Support chip BCM532x series P12 IC

Problems Resolved:

1. Fix: Incorrect SNMP value in Bridge MIB -dot1d STP Port Designated Bridge/Root

2. Fix: STP LBD function but CPU 100% periodically.

3. Fix: factory test issue: DES-3010FL MAC address error.

4. Fix: The source port of DES-3010F SNMP trap was = "0"

5. Fix IGMP Snooping issue - Stop a stream may also stop another stream play in the same port/PC.

Firmware: 2.00.026 (PROM：1.01.005)

Hardware: A1 for DES-3000 series, D1 for DHS-3000 series

Date: Feb. 27, 2006

## Fixed LED issue when reboot

Led bright one times when DUT enter CLI (As R1).

Firmware: 2.00.023 (PROM：1.01.005)

Hardware: A1 for DES-3000 series, D1 for DHS-3000 series

Date: Feb. 21, 2006

Add support new flash:

S29GL032M90TFIR40

W19B320ABT7H

AT49BV322D-70TU

Firmware: 2.00.020 (PROM：1.01.003)

Hardware: A1 for DES-3000 series, D1 for DHS-3000 series

Date: Dec. 26, 2005

Problems Resolved:

- Fix D-Link Russia reported security issue. Correction actions:

* Remove super password

* Change backdoor password to console access only.

2. Fix “trusted_host” IP works in telnet/web, but fails in SNMP problem.

3. Fix WinXP 802.1x client showing wrong message when running “local 802.1x”

4. Fix message error (IP=0.0.0.0) when sending “linkup/down” trap.

5. Fix static FDB table problem when co-working with port-security function.

6. Fix failed to configure “Unicast/Multicast static filtering table” from D-View SNMP tool.

7. Fix Web UI Java script security issue: If using user-level account login from Web UI (eg., Firefox) and then disable Java script of web, that user-level account can have admin-level access right.

8. Fix incorrectly sending “port security access violation trap” issue.

Enhancements:

- DHCP Auto-configuration

- Web UI supports non-IE browser

- STP loopback prevention

- Bandwidth control GUI/CLI message remind

- SMTP client Log enhancement

- Log admin password/IP change activity enhancement

- SIM commander

- CPU interface filtering

- Port Security Violation Trap/Log

Notes:

1. For firmware upgrade from R1 to R2, please disable 802.1x state first. Then re-configure the 802.1x function in R2 firmware version.

2. After upgrade firmware from R1 to R2, user need re-configure the SIM and syslog function. Because these functions be upgraded.

DES-30xx Firmware 1.01.021 (Released: June 15, 2005)

First release version. For features, please check the Data Sheet.