# DGS-3100 Series Firmware Release Note

Firmware: 	v1.00.36 (DGS-3100-24/48)
	v1.00.37 (DGS-3100-24P/48P)

Hardware: A1

Date: July, 2007

Problems Resolved:

- If a BPDU is sent to an edge port with a priority lower than the DUT (assuming the DUT is the root bridge), then the link partner (connected through the edge port) becomes the root bridge ***according to the DUT as well***. The port is still treated as portfast (or edge port in DGS terminology) in the convergence process. (i.e. It is moved automatically to the forwarding state without going through listening/learning).

- The web shows the PoE ports for the first unit only

- It’s possible to create a SNMPv3 user with a group name that wasn’t created and defined yet via the web

- It’s possible to create a SNMPv3 group with a view name that wasn’t created by the user or already exist at the view table

- It’s possible to create a SNMP community with a view name that wasn’t created by the user or already exist at the view table

- The wrong behavior in the WEB when configuring access_profile - if the user create an access profile with a wildcard of "0"s (and any destination) when trying to add an access id you will get an error message , in the CLI the access id will not appear as it should. When configure the same thing via CLI there is no problem. This problem causes problem when trying to download the configuration file to the device

- When using CLI, it’s possible to create SNMP community/groups/users with none existing “view”.

- It is impossible to choose auto_assign option in "config access_profile " command.

- In the CLI, when try to define more then 2 methods for login/enable, the command was not completed when use "tab" or "?"

- In the CLI, it is impossible to configure PVE with LAG uplink

- When typing the command "config stp xyxyxy m" and pressing "enter" - the device gets stuck

- If a BPDU is sent to an edge port with a priority lower than the DUT (assuming the DUT is the root bridge) than the link partner (connected through the edge port) becomes the root bridge *** according to the DUT as well***. The port is still treated as portfast (or edge port in DGS terminology) in the convergence process. (i.e. it is moved automatically to the forwarding state without going through listening/learning).

- The web shows the PoE ports for the first unit only.

- It’s possible to create SNMPv3 user with a group name that wasn’t created and defined yet via the web.

- It’s possible to create SNMPv3 group with a view name that wasn’t created by the user or already exist at the view table.

- The wrong behavior in the WEB when configure access_profile – if the user create an access profile with a wildcard of “0”s (and any destination). 
When trying to add an access ID, you will get an error message, in the CLI, the access ID will not appear as it should. 
When configure the same thing via the CLI, there’s no problem. This problem cause problem when try to download the configuration file to the device.

- When downloading configuration, that sets certificate 2 as active - doesn't configured it at startup.

- After downloading configuration without specifying ip (or dhcp) - device starting up with ip 1.0.0.0

- There should be a "tagged ports" line in the "show vlan" command.

- When sending wire speed traffic using cable length more the 120 meter on port 22 and 46 meter packet loss occurred.

- To reconstract:

1) Set ingress rate limit on port 1 to 3.5 Mbps

config rate_limit 1 3500

2) Sent FWS traffic from SMB to port 1. Frame size is 64.
3) View traffic on port 2 of smartbits - rate is 2.2 Mbps. 2.2*84/64 (for IFG) = 2.9 Mbps

Enhancements:

Support PoE (DGS-3100-24P/48P)

Known Issues:

- In RSTP, edge ports can receive BPDUs that would change the STP port mode, and still be treated as edge ports
[Workaround] edge port is mainly used for the case of connecting device to the switch directly. For DGS-3100, ports are rarely configured to operate in this way.

- It is possible to change the stacking queue method to WRR
[Workaround] the stacking queue has highest priority in default. The user guide can address that stacking performance will be degraded if we change the stacking queue method to WRR.

- It is impossible to activate the third web session 
[Workaround] two web sessions work properly within the stack, in standalone mode there is no problem at all.

- when get connected to telnet/ssh with user with priv-1 (user) cannot going back with priv-15 (admin)
[Workaround] the CLI command, “enable admin”, does not work properly. The user should log out first and then log in as admin.

- An irrelevant reboot message appears after reset the device 
[Workaround] Reset without reboot will only reset the startup configuration while keeping the running configuration. The running configuration will be cleared if the user confirms the reboot message.

- It's possible to configuration Stacking port's (49-50) under LAG
[Workaround] the system will have the same behavior either stack ports configured as LAG or do not.

- In CLI, it’s not possible to create SNMP user assign with a group of v1.
[Workaround] Web GUI has removed the conditional check to support the creation of SNMP user (v3) with a v1 group. Meanwhile, CLI does not remove the conditional check yet.

- In a very specific scenario, the device will not reload with the default IP address, this is the scenario:

Open a CLI connection

See that the IP address on the device is the default one - 10.90.90.90/24

1. Execute reset and select [N] when you asked whether to reboot the device.

2. Execute reboot.

3. Once the device is reloaded execute “show switch”, see that the ip is 0.0.0.0

4. See that the configuration is empty.
The CLI script:
[Workaround] The bug happens only with the above specific steps.

=================================================================