Doc. No: KI0028

Date: 2009/Jun/02

Known Issue – New Release FW bugs

DNS-722-4/ DNS-726-4

Model:

- DNS-722-4

- DNS-726-4

H/W Ver.：

- DNS-722-4 A1

- DNS-726-4 A1

F/W Ver:

- DNS-722-4 A1 v1.00 (1.00b8(1.00.0506.2009))

- DNS-726-4 A1 v1.00 (1.00b8(1.00.0506.2009))

There were some problems in the first release firmware with occurrence possibility which were found at the function test in DHQ lab.

Issue Description

- IE6 would encounter a problem to close:

After install the live view or playback ActiveX component, the Internet Explorer version 6 (IE6) of Windows XP English version (SP3) will auto close program when click on any other links. But it wouldn't happen on IE7.

Root Cause

ActiveX component compatibility issue:

It is caused by the ActiveX conflict with certain program or browser plug-in components.

Solution Status to Fix the Problems:

Try to find out the conflict program or components and avoid the problem. This will be fixed in new version.

Suggestion

Please update IE6 to IE7.

- Web UI locked during RAID building:

If first admin login and enter the maintenance page to build a RAID volume, and second user use “admin” to login again and do the same thing will cause the RAID build fail.

Root Cause

Multiple admin user login issue

When building RAID, the web server closes the admin login session.

Solution Status to Fix the Problems:

Firmware need to block the second admin login when first admin is doing some actions. This will be fixed in new version.

Suggestion

Please don’t login NVR twice and change the RAID function.

- Unable to get IP from DHCP router sometimes:

This issue happens sometimes especially after NVR reboot and there is more than 1 NVR in the network.

Root Cause

DHCP client waiting time issue

The DHCP client waiting time is too short in certain reboot operations (ex.: format HDD, RAID, restore factory default…).

Solution Status to Fix the Problems:

Find out what kind of operation will cause this issue and extend the DHCP client waiting time. This will be improved in new version.

Suggestion

1. When NVR can’t get IP from DHCP, the IP would be a default IP (192.168.0.32). Please use “NVR search” utility to search NVR and set the IP manually or enable DHCP again.

- The devices that searched by UPNP are too less or different every time:

Camera search use UPnP to search network cameras in the network. Every camera has different response time of UPnP request and especially when there are over 16 cameras in the network, the camera list will display fewer than actual camera quantity.

Root Cause

UPnP search timing issue

Since UPnP is not a confirm-base protocol, it broadcast the query and only display those devices which response.

Solution Status to Fix the Problems:

Extend the UPnP search time and increase request time. This will be improved in new version.

Suggestion

User can use camera search tools to find camera and input the camera IP manually at the “Setup>Camera Setup” page.

- Model name auto-detection fail:

The auto detection function is used when manually input camera IP, NVR can help user detect the camera model. But some models can’t response the auto detection correctly. (DCS-3110, DCS-3415, DCS-5610, DCS-6110, DCS-6111)

Root Cause

Model detection response error

The CGI/SDK integration has problem.

Solution Status to Fix the Problems:

Update the CGI/SDK in the NVR firmware. This will be fixed in new version.

Suggestion

User can use camera search tools to find camera and select the camera model manually at the “Setup>Camera Setup” page.