The following instructions describe how to adding printers under Mac OS X Tiger (10.4) with various printing protocols, such as AppleTalk, Bonjour, Internet Printing Protocol (IPP), Line Printer Daemon (LPD), and HP Jet Direct – Socket in order to print through Print Server.

After attaching the Print Server to your network using the directions provided in the Quick Installation Guide, change the IP Address of your Macintosh to access the Print Server’s Web configuration.

Network Configuration

Click this icon at the bottom to open your System Preferences window as below.

After clicking Network icon from System Preferences window to enter the window below, select TCP/IP folder. Owing to the default IP Address of the Print Server is 192.168.0.10, you need to manually change your IP Address to 192.168.0.x, where x is any number between 1 and 254 (except 10 which is the IP Address of the Print Server.) Therefore, you need to select Manually option from Configure IPv4 pull-down menu in order to change your IP Address.

Suppose that the IP Address of the Printer Server is 10.67.5.135, you may manually change the IP Address of your Macintosh as 10.1.1.1. The IP Addresses and Subnet Mask shown here are examples only.

After done the TCP/IP configuration, switch to AppleTalk folder to enable the AppleTalk by checking Make AppleTalk Active in this window. Then click Apply Now button to implement the new setting and exit this window.

Adding Printer

Click Go menu at the top tool bar and select Applications option.

When the Utilities window shows up as below, click  and select  to open Printer List window. Since no printer is set up on your Macintosh so far, you will be prompted to add a printer. Click Add button on the pop-up dialogue box to open Printer Browser window.

Mac OS X Tiger (10.4) supports five printing protocols: (1) AppleTalk, (2) Bonjour, (3) Internet Printing Protocol (IPP), (4) Line Printer Daemon (LPD), and (5) HP Jet Direct – Socket. To add an AppleTalk-enable or Bonjour-enable printer, click on the  button in the Printer Browser window. Otherwise, click on the  button to add a printer with other protocols.

### Adding an AppleTalk-enabled printer

You may check the printer setting of the Printer Server’s Web configuration, and select the same printer model from Printer Name list on Printer Browser window. For the following example, select PS-C0D26A-U3. Then the Printer Browser system will start its Auto Select; otherwise, you may manually select your printer at Print Using drop-down menu by brand and model. To exit the Printer Browser window with implementing the new setting, click Add button.

Adding a Bonjour-enabled printer

The procedures about adding a Bonjour-enabled printer to your Macintosh are same as AppleTalk.

When the printer you just added shows up in the Printer List window, you can now quit the Printer Setup Utility and start printing.

Adding an IP printer

If you want to set up a printer with other protocols, click on the  button in the Printer Browser window. Select a desired printing protocol from the Protocol drop-down menu.

After done the printing protocol selection, type the IP Address of the printer, which will also be synchronously created in the Name field, in the Address field. The Printer Browser will indicate that it is verifying the address. If the printer is found, the Printer Browser will state “Valid and complete address” under the Address field.

The printer utility is able to detect the printer and may automatically select the correct printer driver in the Print Using field. Likewise, you may manually select the printer-maker from the Print Using drop-down menu, and then select printer model from the scroll-down list, or select Generic PostScrip Printer option from the Print Using drop-down menu if the model is not listed. To implement the setting, click Add button.

The setup procedures for these three printing protocols (Internet Printing Protocol (IPP), Line Printer Daemon (LPD), and HP Jet Direct – Socket) are similar. See also the following illustrations. The IP addresses and Names shown in the illustrations are examples only.

For set up IPP Printing:

For set up LPD Printing:

For set up HP Printing:

If the printer you want to add is a HP, select HP Jet Direct – Socket at Protocol drop-down menu. Enter the IP Address of this HP printer in the Address field, and click Add button after done the section of printer model in the Print Using drop-down menu.

When the Installable Options pop-up window shows up as below, click Continue button.

The new adding printers will be displayed in the Printer List window. You may continue to add other printers by clicking the Add button, or quit the Print Setup Utility and start printing.