VPN L2TP Application

Installation Guide

Configuring a Remote Access L2TP VPN Dial-in Connection

A remote worker establishes a L2TP VPN connection with the head office using Microsoft's VPN Adapter (included with Windows XP/2000/ME, etc.). The router is installed in the head office, connected to a couple of PCs and Servers.

Dial-in

Configuring L2TP VPN in the Office

The input IP address 192.168.1.200 will be assigned to the remote worker. Please make sure this IP is not used in the Office LAN.

1

4

3

2

5

6

| Item | Function |  | Description |
| --- | --- | --- | --- |
| 1 | Connection Name | YUJI_test | Given a name of L2TP connection |
| 2 | Dial in |  | Check Dial in |
|  | Private IP Address Assigned to Dialing User | 192.168.15.199 | An assigned IP address for the remote worker |
| 3 | Username | yuji | Input username & password to authenticate remote worker |
|  | Password | 123456 |  |
| 4 | Auth.Type | Chap(Auto) | Keep as default value in most of the cases. |
| 5 | Idle Timeout | 0 | The connection will be disconnected when there Is no traffic in a predefined period of time. Idle time 0 means the connection is always-on. |
| 6 | IPSec |  | Enable for enhancing your L2TP VPN security. |
|  | Authentication | MD5 | Both sites should use the same value. |
|  | Encryption | 3DES |  |
|  | Perfect Forward Secrecy | None |  |
|  | Pre-shared Key | 12345678 |  |

Configuring Remote Worker for Dial-out to the Router

| Step 1 Select “New Connection Wizard” to create a VPN-L2TP dial out service. |  |
| --- | --- |
| Step 2 Click Next > to the next step. |  |
| Step 3 Check “Connect to the network at my workplace” radio button. Click Next > to the next step. |  |
| Step 4 Check “Virtual Private Network connection” radio button. Click Next > to the next step. |  |
| Step 5 Give a name to the L2TP connection. Click Next > to the next step. |  |
| Step 6 Input VPN-L2TP Server’s IP address. You can use Host name, only if the DDNS function is enabled in the Router. Refer to the user manual for more detail. Click Next > to the next step. |  |
| Step 7 Click Finish to the next step. |  |
| Step 8 Open Network Connections and double click on the VPN L2TP icon. |  |
| Step 9 Select “Networking” tab, and change “Type of VPN” to “L2TP IPSec VPN”. |  |
| Step 10 Select “Security” tab. Click IPSec Settings… to the next step. |  |
| Step 11 Input pre-shared key. Note: This pre-shared key must be identical as set up in the Router. Click OK to the next step. |  |
| Step 12 Click OK to continue. |  |
| Step 13 User name and Password should be identical as set up in Router. (See Item 3) Click Connect to establish a connection to your VPN-L2TP Server. Completed! |  |