| D-Link DWA-520 Wireless Card Test / Verification Report Product Model; DWA-520. Adapter Interface; PCI. Driver Version; 4.1.2.725 (98/ME) 5.3.0.46 (2K/XP) 4.2.2.51 (XP 64) 7.6.0.1721 (Vista x86) 7.6.0.1721 (Vista x64). Utility Version; 1.20. Setup Version; S0017. Build Version; 4.3.1.1021. Channel Region; Worldwide. Hardware Version; A1. PCB Version; 4A1G. Product Key Component; Atheros AR2414. No. 8 Li-Shing Rd. 7, Science-based Industrial Park, Hsinchu, Taiwan, R.O.C. Tel : 886-3-5636666 |
| --- |
| Approver | Reviser | Department | Writer |
| Margaret Lee #3626 Magaret_Lee@alphanetworks.com | Margaret Lee #3626 Magaret_Lee@alphanetworks.com | TLD2 | Josh Chen #3504 Josh_Chen@alphanetworks.com |

Product Features

| Products Features | Detail |
| --- | --- |
| Adapter Interface | PCI |
| Operation System Support | Window 98/ME Windows 2000/XP/XP 64-bit Windows Vista x86/x64 |
| Dynamic Data Rate Scaling for IEEE 802.11b | 1, 2, 5.5 and 11 Mbps |
| Dynamic Data Rate Scaling for IEEE 802.11g | 6, 9, 12, 18, 24, 36, 48 and 54 Mbps |
| Channel Region | - 802.11b/g: FCC: Ch. 1 ~ 11 ETSI: Ch. 1 ~ 13 |
| Encryptions | - 64/128/152-bit WEP (152-bit WEP is not support under Vista x86/x64) - WPA/WPA2 PSK and EAP (Cipher Modes: TKIP and AES) - 802.1X EAP |
| WPS function | Yes (Excluding 98/ME) |
| Qos WMM | Yes (Excluding 98/ME) |
| WMM Power Saving | N/A |
| XR function | Yes |
| Super G function | - Super G without Turbo function |
| Multilingual UI | 7 languages |
| Multilingual WPS UI | 7 languages |
| RoHs Compliant | Yes |
| Wi-Fi Certification | Yes |
| Microsoft WHQL/DTM Certification | Yes |

Estimation

The test request note is shown as the following file:

10/20 Lab starts a new test setup packet S0016. These drivers supports Windows 98, Me , XP, XP-64, Vista x86 and Vista x64. In order to avoid the Dynamic Turbo Mode bug under Vista, RD update the setup packet to S0017 at 10/21, which disable the Dynamic Turbo function in Vista x86/64. D-Link has conditional approved it. Atheros will fix this bug in next version. In this test, the Win 98,ME, XPx86 and XP x64 drivers do not change, so we focus on Vista x86 and Vista x64 driver only. During the test, no bug was found and no bug was left.

Table of Contents

1. Basic Function Test	12

1-1 Wireless Band	12

1-2 Connectivity Test	12

1-3 Disable/Enable Test	13

1-4 OS Standby and Hibernation Test	13

1-5 Wi-Fi Protected Setup (WPS) Function	13

1-6 Windows Native Configuration Utility	13

1-7 Driver Installation and Un-installation	13

2. Advanced Function Test	14

2-1 WPA-EAP Encryption	14

2-2 WPA2-EAP Encryption	14

2-3 802.1X EAP Encryption	14

2-4 WMM Function	15

2-5 Roaming	15

2-6 Driver Modification Protection for Normal Users	15

2-7 Microsoft WHQL Certification	15

3. UI Options Support	16

3-1 UI Options	16

3-2 Multilingual Utility and WPS UI Test	18

4. Throughput Test	19

4-1 Infrastructure Mode	19

4-4-1 Windows XP	19

4-1-2 Windows XP x64	19

4-1-3 Windows Vista x86	20

4-1-4 Windows Vista x64	20

4-2 Infrastructure Mode in Different Channels	21

4-2-1 Windows XP	21

4-2-2 Windows XP x64	21

4-2-3 Windows Vista x86	21

4-2-4 Windows Vista x64	21

4-3 Throughput Test in Super G Mode	22

4-3-1 Windows XP	22

4-3-2 Windows XP x64	22

4-3-3 Windows Vista x86	22

4-3-4 Windows Vista x64	22

4-4 Ad-Hoc Mode	23

4-4-1 Windows XP	23

4-4-2 Windows XP x64	23

4-4-3 Windows Vista x86	23

4-4-4 Windows Vista x64	23

5. Endurance Test	24

5-1 Infrastructure Mode	24

5-2 Ad-Hoc Mode	24

Appendix PC Compatibility	25

PC Compatibility	25

# 1. Basic Function Test

Test Environment

Station: ASUS P5B-VM [OS: Vista x64], ASUS M2N-E [OS: Vista x86]

AP: DWL-8200 [F/W: 2.04], DAP-1522 [F/W: 1.10]

Test Place: Open Site

Test Procedure: Let DUT connects to AP and check the connection status by Ping command.

## 1-1 Wireless Band

| Wireless Band | SSID | Result (Pass/ Fail) |
| --- | --- | --- |
| 802.11b only | Any ASCII string with 32 characters length | Pass |
| 802.11g only | STA SSID :12345678901234567890123456789012 AP SSID: 12345678901234567890123456789012 | Pass |
| 802.11 b/g mixed | STA: 1234567890!@#$%^&*(){}¦:”<>?~`/\ AP: 1234567890!@#$%^&*(){}¦:”<>?~`/\ | Pass |

## 1-2 Connectivity Test

Abbreviation

WB = Wireless Band, Ch. = Channel, KI = Key Index, AH = Ad-Hoc, Hex. = Hexadecimal, ASC. = ASCII

- DUT is set to DHCP client mode during all tests.

| Item | WB | Ch. | Authentication | Cipher | KI | Result (Pass/ Fail) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 11g | 1 | Open System | 64-bit Hex. | 1 | Pass |
| 2 | 11g | 2 | Open System | 128-bit Hex. | 2 | Pass |
| 3 | 11g | 3 | Open System | 64-bit ASC. | 3 | Pass |
| 4 | 11g | 4 | Open System | 128-bit ASC. | 4 | Pass |
| 5 | 11g | 5 | Shared Key | 64-bit Hex. | 1 | Pass |
| 6 | 11g | 6 | Shared Key | 128-bit Hex. | 2 | Pass |
| 7 | 11g | 7 | Shared Key | 64-bit ASC. | 3 | Pass |
| 8 | 11g | 8 | Shared Key | 128-bit ASC. | 4 | Pass |
| 9 | 11g | 9 | WPA-PSK(63) | TKIP | N/A | Pass |
| 10 | 11g | 10 | WPA-PSK(63) | AES | N/A | Pass |
| 11 | 11g | 11 | WPA2-PSK(63) | TKIP | N/A | Pass |
| 12 | 11g | 12 | WPA2-PSK(63) | AES | N/A | Pass |
| 13 | 11g | 13 | Open System | 152-bit / No WEP | N/A | Pass |
| 14 | 11g AH | 1 | Open System | No WEP | N/A | Pass |
| 15 | 11g AH | 6 | Open System | 128-bit Hex. | 1 | Pass |
| 16 | 11g AH | 13 | Open System | 128-bit ASC. | 4 | Pass |

## 1-3 Disable/Enable Test

| Result Item | 98SE | ME | 2K SP4 | XP SP3 | XP x64 | Vista x86 | Vista x64 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Disable/Enable DUT | Pass | Pass | Pass | Pass | Pass | Pass | Pass |

## 1-4 OS Standby and Hibernation Test

| OS | Standby (Pass/Fail) | Hibernation (Pass/Fail) |
| --- | --- | --- |
| Windows 98SE | Pass | N/A |
| Windows ME | Pass | N/A |
| Windows 2000 SP4 | Pass | Pass |
| Windows XP SP3 | Pass | Pass |
| Windows XP x64 | Pass | Pass |
| Windows Vista x64 | Pass | Pass |
| Windows Vista x86 | Pass | Pass |

## 1-5 Wi-Fi Protected Setup (WPS) Function

- WPS function doesn’t support in 98/ME

- DAP-1522 [F/W: 1.10], DIR-655 [F/W: 1.05]

| Operating System | Push Button (Pass / Fail) | PIN Code (Pass /Fail) |
| --- | --- | --- |
| Windows 2000 SP4 | Pass | Pass |
| Windows XP SP3 | Pass | Pass |
| Windows XP x64 | Pass | Pass |
| Vista x86 SP1 | Pass | Pass |
| Vista x64 SP1 | Pass | Pass |

## 1-6 Windows Native Configuration Utility

| OS Mode | Vista x86 | Vista x64 |
| --- | --- | --- |
| Infrastructure | Pass | Pass |
| Ad-Hoc | Pass | Pass |

## 1-7 Driver Installation and Un-installation

| Operating System | Install by Setup (Pass/Fail) | Install manually (Pass/Fail) | Uninstall by Setup (Pass/Fail) | Uninstall Manually (Pass/Fail) |
| --- | --- | --- | --- | --- |
| Windows 98SE | Pass | Pass | Pass | Pass |
| Windows ME | Pass | Pass | Pass | Pass |
| Windows 2000 SP4 | Pass | Pass | Pass | Pass |
| Windows XP SP3 | Pass | Pass | Pass | Pass |
| Windows XP x64 | Pass | Pass | Pass | Pass |
| Vista x86 SP1 | Pass | Pass | Pass | Pass |
| Vista x64 SP1 | Pass | Pass | Pass | Pass |

# 2. Advanced Function Test

Test Environment

Station: ASUS P5B-VM [OS: Vista x64], ASUS M2N-E [OS: Vista x86]

AP: DAP-1522 [F/W: 1.10], Cisco 1200 [F/W: 12.3(2) JA]

Test Tools: Airopeek NX 2.0.1 with WME extension

## 2-1 WPA-EAP Encryption

- EAP-TTLS authentications are not supported by Windows Vista API.

| Item | Radius (Odyssey/IAS) | Cipher (TKIP/AES/Auto) | EAP | Result (Pass/Fail) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Vista x86/x64 | Other OS |
| 1 | IAS | TKIP | EAP-TLS | Pass | Pass |
| 2 | IAS | AES | PEAP(EAP-MSCHAPV2) | Pass | Pass |
| 3 | Odyssey | TKIP | EAP-TTLS(PAP) | N/A | Pass |
| 4 | Odyssey | AES | EAP-TTLS(MSCHAP) | N/A | Pass |
| 5 | Odyssey | Auto | EAP-TTLS(MSCHAPV2) | N/A | Pass |

## 2-2 WPA2-EAP Encryption

- EAP-TTLS authentications are not supported by Windows Vista API.

| Item | Radius (Odyssey/IAS) | Cipher (TKIP/AES/Auto) | EAP | Result (Pass/Fail) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Vista x86/x64 | Other OS |
| 1 | IAS | AES | EAP-TLS | Pass | Pass |
| 2 | IAS | TKIP | PEAP(EAP-MSCHAPV2) | Pass | Pass |
| 3 | Odyssey | Auto | EAP-TTLS(PAP) | N/A | Pass |
| 4 | Odyssey | AES | EAP-TTLS(MSCHAP) | N/A | Pass |
| 5 | Odyssey | TKIP | EAP-TTLS(MSCHAPV2) | N/A | Pass |

## 2-3 802.1X EAP Encryption

- LEAP and EAP-TTLS authentications are not supported by Windows Vista API.

- The UI only supports 802.1X Dynamic WEP authentication mode.

| Item | Radius (Odyssey/IAS) | EAP-Type | Result (Pass/Fail) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Vista x86/x64 | 2K/XP |
| 1 | IAS | PEAP (EAP-MSCHAPv2) | Pass | Pass |
| 2 | IAS | EAP-TLS | Pass | Pass |
| 3 | Odyssey | LEAP | N/A | Pass |
| 4 | Odyssey | EAP-TTLS(PAP) | N/A | Pass |
| 5 | Odyssey | EAP-TTLS(MSCHAP) | N/A | Pass |
| 6 | Odyssey | EAP-TTLS(MSCHAPV2) | N/A | Pass |

## 2-4 WMM Function

| OS | Driver Version | Band | Result (Pass/Failed) |
| --- | --- | --- | --- |
| XP SP3 | 5.3.0.46 | 802.11b | Pass |
| XP SP3 | 5.3.0.46 | 802.11g | Pass |
| XP 64 SP2 | 4.2.2.51 | 802.11b | Pass |
| XP 64 SP2 | 4.2.2.51 | 802.11g | Pass |
| Vista x86 SP | 7.6.0.1721 | 802.11b | Pass |
| Vista x86 SP1 | 7.6.0.1721 | 802.11g | Pass |
| Vista x64 SP1 | 7.6.0.1721 | 802.11b | Pass |
| Vista x64 SP1 | 7.6.0.1721 | 802.11g | Pass |

## 2-5 Roaming

| Description | Result (Pass/ Fail) |
| --- | --- |
| Test a station’s ability to roam between two access points | Pass |

## 2-6 Driver Modification Protection for Normal Users

| Operating System | Result (Pass/Fail) |
| --- | --- |
| 2000 SP4 | Pass |
| XP SP3 | Pass |
| XP 64 SP2 | Pass |
| Vista x86 SP1 | Pass |
| Vista x64 SP1 | Pass |

## 2-7 Microsoft WHQL Certification

| OS | Driver Ver. | Submission ID | Result (Pass/Fail) |
| --- | --- | --- | --- |
| Windows 2000/XP | 5.3.0.46 | 1251532 | Pass |
| Windows XP 64 | 4.2.2.51 | 1222383 | Pass |
| Vista x86/x64 | 7.6.0.1721 | 1309414 | Pass |

# 3. UI Options Support

Test Environment

Station: ASUS P5B-VM [OS: Vista x64], ASUS M2N-E [OS: Vista x86]

AP: DWL-8200 [F/W: 2.04], DAP-1522 [F/W: 1.10]

Multilingual UI Test Method: Check whether the Install-Shield, UI and Connection Wizard correspond to current language version of Windows XP SP3.

## 3-1 UI Options

| Description | Result (Pass/Fail) |
| --- | --- |
| Connect Wizard: |  |
| Manual connect to a wireless network. | Pass |
| Connect to a wireless network using WPS | Pass |
| WPS Wizard: |  |
| PBC (Push Button Configuration) | Pass |
| PIN (Personal Identification Number) | Pass |
| Press Button (Push Button Mode) | Pass |
| Wireless Networks: |  |
| Refresh | Pass |
| WPS | Pass |
| Connect | Pass |
| Activate | Pass |
| My Wireless Networks |  |
| New | Pass |
| Modify | Pass |
| Delete | Pass |
| Activate | Pass |
| Profile Settings: |  |
| Basic Settings: |  |
| Profile Name | Pass |
| SSID | Pass |
| Network Type (Infrastructure) | Pass |
| Network Type (Ad hoc) | Pass |
| Set Security Option: |  |
| None: |  |
| No Security | Pass |
| WEP: |  |
| Auto | Pass |
| Open | Pass |
| Shared | Pass |
| Key Index (key 1, 2, 3, 4) | Pass |
| WPA/WPA2-Personal: |  |
| Auto | Pass |
| TKIP | Pass |
| AES | Pass |

| Description | Result (Pass/Fail) |
| --- | --- |
| WPA/WPA2-Enterprise: |  |
| Auto | Pass |
| TKIP | Pass |
| AES | Pass |
| Advanced Config | Pass |
| 802.1x: |  |
| Advanced Config | Pass |
| Profile Details: |  |
| Network Name | Pass |
| Network Type | Pass |
| Security | Pass |
| Advanced Config in EAP Methods: |  |
| EAP-Type | Pass |
| EAP-Type -> Subtype | Pass |
| User Certificate | Pass |
| Validate Server Certificate | Pass |
| Username | Pass |
| Domain Name | Pass |
| Password | Pass |
| Confirm Password | Pass |
| TTLS Identify |  |
| Add | Pass |
| Remove | Pass |
| Support  About: |  |
| Driver Version | Pass |
| Utility Version | Pass |
| Mac Address | Pass |
| Build Info: |  |
| Build Version | Pass |
| Reg. Domain | Pass |
| Firmware | Pass |
| Setup Version | Pass |

## 3-2 Multilingual Utility and WPS UI Test

| Item | Country | Result (Pass/Failed) |
| --- | --- | --- |
| 1 | English | Pass |
| 2 | French | Pass |
| 3 | Portuguese | Pass |
| 4 | Russian | Pass |
| 5 | Spanish | Pass |
| 6 | Simplified Chinese | Pass |
| 7 | Traditional Chinese | Pass |

# 4. Throughput Test

Test Environment

Server: Toshiba Tecra A3/S2 [OS: XP SP3]

Station: ASUS M2N-E [OS: Vista x86/x64 SP1]

Test Method: Run a single TX or RX pair with “Throughput Script” for 60 seconds

Test Tool: Chariot Console 5.4.0, Chariot Endpoint 6.60.42

Default Channel: 6

Place: Shield Room E

## 4-1 Infrastructure Mode

### 4-4-1 Windows XP

Unit: Mbps

| AP Settings Encryptions | Mode | DWL-8200 | Cisco 1200 | Cameo DI-624 D2 |
| --- | --- | --- | --- | --- |
|  |  | 2.00 | 12.3(2)JA | 4.04TW |
| No Encryption | TX | 21.932 | 20.661 | 22.124 |
|  | RX | 22.314 | 21.547 | 21.135 |
| WPA-PSK TKIP | TX | 20.920 | 20.415 | 20.045 |
|  | RX | 22.206 | 20.820 | 21.270 |
| WPA2-PSK AES | TX | 21.274 | 20.070 | 22.040 |
|  | RX | 22.252 | 21.147 | 21.513 |

### 4-1-2 Windows XP x64

Unit: Mbps

| AP Settings Encryptions | Mode | DWL-8200 | DIR-300 | DAP-1522 | DIR-655 |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.04 | 1.04 | 1.10 | 1.21 |
| No Encryption | TX | 19.120 | 18.507 | 21.305 | 21.649 |
|  | RX | 20.224 | 20.143 | 24.526 | 22.015 |
| Open System + 128-bit WEP | TX | 18.301 | 18.278 | 21.207 | 21.662 |
|  | RX | 19.399 | 19.825 | 24.182 | 22.138 |
| WPA-PSK TKIP | TX | 18.193 | 18.182 | 21.359 | 21.424 |
|  | RX | 18.334 | 19.652 | 23.924 | 21.879 |
| WPA2-PSK AES | TX | 18.125 | 19.882 | 21.162 | 21.683 |
|  | RX | 18.214 | 20.341 | 22.878 | 21.939 |

### 4-1-3 Windows Vista x86

Unit: Mbps

| AP Settings Encryptions | Mode | DWL-8200 | DIR-300 | DAP-1522 | DIR-655 |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.04 | 1.04 | 1.10 | 1.21 |
| No Encryption | TX | 22.092 | 20.366 | 21.932 | 21.362 |
|  | RX | 23.412 | 20.687 | 23.072 | 21.385 |
| Open System + 128-bit WEP | TX | 1.627 | 20.825 | 21.529 | 21.343 |
|  | RX | 23.284 | 19.743 | 24.483 | 22.166 |
| WPA-PSK TKIP | TX | 21.288 | 20.035 | 21.337 | 21.102 |
|  | RX | 23.221 | 19.319 | 24.352 | 22.246 |
| WPA2-PSK AES | TX | 21.879 | 20.384 | 21.863 | 21.735 |
|  | RX | 23.292 | 20.741 | 23.156 | 22.050 |

### 4-1-4 Windows Vista x64

Unit: Mbps

| AP Settings Encryptions | Mode | DWL-8200 | DIR-300 | DAP-1522 | DIR-655 |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.04 | 1.04 | 1.10 | 1.21 |
| No Encryption | TX | 22.020 | 20.322 | 21.805 | 21.523 |
|  | RX | 23.251 | 20.598 | 24.982 | 22.354 |
| Open System + 128-bit WEP | TX | 22.331 | 20.436 | 21.743 | 21.303 |
|  | RX | 23.150 | 19.313 | 23.575 | 22.412 |
| WPA-PSK TKIP | TX | 21.522 | 20.378 | 21.821 | 21.766 |
|  | RX | 22.695 | 20.288 | 24.069 | 22.318 |
| WPA2-PSK AES | TX | 221.212 | 20.239 | 21.751 | 22.249 |
|  | RX | 22.872 | 20.717 | 24.386 | 21.907 |

## 4-2 Infrastructure Mode in Different Channels

### 4-2-1 Windows XP

Unit: Mbps

| AP | Channel | Open (WEP OFF) |  | WPA2-PSK (AES) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TX | RX | TX | RX |
| Cisco 1200 12.3(2)JA | 1 | 20.625 | 20.952 | 19.933 | 20.677 |
|  | 6 | 20.580 | 20.940 | 20.536 | 20.717 |
|  | 11 | 20.162 | 20.736 | 19.941 | 20.021 |

### 4-2-2 Windows XP x64

Unit: Mbps

| AP | Channel | Open (WEP OFF) |  | WPA2-PSK (AES) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TX | RX | TX | RX |
| DIR-655 (1.21) | 1 | 21.517 | 22.003 | 20.910 | 22.053 |
|  | 6 | 21.649 | 22.015 | 21.683 | 21.939 |
|  | 11 | 21.676 | 21.089 | 20.138 | 21.658 |

### 4-2-3 Windows Vista x86

Unit: Mbps

| AP | Channel | Open (WEP OFF) |  | WPA2-PSK (AES) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TX | RX | TX | RX |
| DIR-655 (1.21) | 1 | 21.820 | 22.785 | 22.106 | 22.661 |
|  | 6 | 21.362 | 21.385 | 21.735 | 22.050 |
|  | 11 | 21.632 | 22.744 | 21.579 | 22.231 |

### 4-2-4 Windows Vista x64

Unit: Mbps

| AP | Channel | Open (WEP OFF) |  | WPA2-PSK (AES) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TX | RX | TX | RX |
| DIR-655 (1.21) | 1 | 22.272 | 22.831 | 21.967 | 22.700 |
|  | 6 | 21.523 | 22.354 | 22.249 | 21.907 |
|  | 11 | 21.632 | 22.371 | 21.582 | 22.371 |

## 4-3 Throughput Test in Super G Mode

### 4-3-1 Windows XP

Unit: Mbps

| AP | Super G Mode | No WEP |  | WPA-PSK (TKIP) |  | WPA2-PSK (AES) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TX | RX | TX | RX | TX | RX |
| DWL-8200 (2.00) | without Turbo | 25.421 | 28.999 | 25.908 | 29.186 | 24.864 | 27.624 |
|  | Dynamic Turbo | 30.983 | 49.196 | 32.941 | 42.099 | 31.067 | 47.393 |
| Cameo DI-624 D2 (4.04TW) | without Turbo | 24.523 | 28.275 | 24.157 | 27.777 | 24.285 | 27.995 |
|  | Dynamic Turbo | 32.859 | 34.595 | 29.806 | 30.061 | 31.193 | 31.136 |

### 4-3-2 Windows XP x64

Unit: Mbps

| AP | Super G Mode | No WEP |  | WPA-PSK (TKIP) |  | WPA2-PSK (AES) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TX | RX | TX | RX | TX | RX |
| DWL-8200 (2.04) | without Turbo | 27.850 | 25.661 | 27.648 | 24.958 | 27.810 | 26.462 |
|  | Dynamic Turbo | 46.113 | 41.764 | 44.037 | 39.124 | 45.271 | 41.940 |
| Cameo DI-624 D2 (4.04TW) | without Turbo | 29.269 | 28.272 | 28.861 | 27.931 | 28.837 | 28.151 |
|  | Dynamic Turbo | 42.164 | 35.205 | 41.905 | 34.361 | 42.250 | 34.768 |

### 4-3-3 Windows Vista x86

Unit: Mbps

| AP | Super G Mode | No WEP |  | WPA-PSK (TKIP) |  | WPA2-PSK (AES) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TX | RX | TX | RX | TX | RX |
| DWL-8200 (2.04) | without Turbo | 27.283 | 29.841 | 27.375 | 29.485 | 27.335 | 29.268 |
| DI-624 D2 (4.04TW) | without Turbo | 27.437 | 29.292 | 27.551 | 28.614 | 27.916 | 28.812 |

### 4-3-4 Windows Vista x64

Unit: Mbps

| AP | Super G Mode | No WEP |  | WPA-PSK (TKIP) |  | WPA2-PSK (AES) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TX | RX | TX | RX | TX | RX |
| DWL-8200 (2.04) | without Turbo | 27.420 | 29.624 | 26.941 | 29.165 | 27.133 | 29.051 |
| DI-624 D2 (4.04TW) | without Turbo | 28.232 | 29.529 | 27.878 | 28.546 | 27.076 | 28.021 |

## 4-4 Ad-Hoc Mode

### 4-4-1 Windows XP

Unit: Mbps

| Wireless Card | Version | WEP OFF |  | WEP 128-bit |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TX | RX | TX | RX |
| ##### Cisco CB21AG | 3.0.0.104 | 5.885 | 6.007 | 5.529 | 5.944 |
| Buffalo WLI-CBG54 | 4.100.15.5 | 5.084 | 5.704 | 5.054 | 5.670 |

### 4-4-2 Windows XP x64

Unit: Mbps

| Wireless Card | Version | WEP OFF |  | WEP 128-bit |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TX | RX | TX | RX |
| ##### Cisco CB21AG | 4.2.0.297 | 6.124 | 5.806 | 6.013 | 5.821 |
| Buffalo WLI-CBG54 | 4.100.15.5 | 5.688 | 6.361 | 5.583 | 6.258 |

### 4-4-3 Windows Vista x86

Unit: Mbps

| Wireless Card | Version | WEP OFF |  | WEP 128-bit |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TX | RX | TX | RX |
| ##### Cisco CB21AG | 4.2.0.297 | 6.116 | 5.939 | 6.025 | 5.953 |
| Buffalo WLI-CBG54 | 4.100.15.5 | 17.095 | 21.462 | 17.105 | 21.250 |

### 4-4-4 Windows Vista x64

Unit: Mbps

| Wireless Card | Version | WEP OFF |  | WEP 128-bit |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TX | RX | TX | RX |
| ##### Cisco CB21AG | 4.2.0.297 | 6.130 | 5.996 | 6.085 | 5.949 |
| Buffalo WLI-CBG54 | 4.100.15.5 | 17.358 | 21.521 | 16.736 | 21.385 |

# 5. Endurance Test

Test Environment

Test tool: Chariot 5.4, Serv-U 2.5, FTP Command

Test Methods: FTP => Run a 600MB TX and 600MB RX simultaneously in open side over 12 hours.

Chariot => Run a 600MB TX and a 600MB RX Chariot test pair in shield room over 12 hours.

Test Place: Lab (Open Side), Chamber (Shield Room)

## 5-1 Infrastructure Mode

| Item | PC/Note Book | OS | AP | Encryption | Channel | Method | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ASUS M2N-E | Vista x86 | DAP-1522 | WPA2-PSK AES | 3 | FTP | Pass |
| 2 | ASUS P5B-VM | Vista x64 | C300BRS4v2 | WPA-PSK TKIP | 9 | FTP | Pass |
| 3 | ASUS P5B-VM | Vista x86 | DIR-605 | WPA2-PSK AES | 2 | FTP | Pass |
| 4 | ASUS M2N-E | Vista x64 | DIR-655 | WPA2-PSK AES | 6 | Chariot | Pass |

## 5-2 Ad-Hoc Mode

| Item | PC/Note Book | OS | Test Card | Encryption | Channel |  | Method | Result (Pass/Fail) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ASUS M2N-E | Vista 86 | DWA-520 | WEP-128 bit HEX |  | 11 | FTP | Pass |
|  | ASUS P4B | XP SP3 | WLUSB300AGN |  |  |  |  |  |
| 2 | ASUS M2N-E | Vista 86 | DWA-520 | WEP-128 bit HEX |  | 3 | FTP | Pass |
|  | ASUS P5B-VM | Vista x64 | DWA-520 |  |  |  |  |  |

# Appendix PC Compatibility

## PC Compatibility

| Ser No. | Model Name | CPU | BIOS | MB Chipset | BUS | USB Chip | OS | Test Conclusion (Pass/Fail) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TLD2120 | ASUS P5B-VM | P5B-VM | AMI | Intel G965 | PCI (USB2.0) | Intel ICH8 | Vista x86 Vista x64 | Put 600MB | Pass |
|  |  |  |  |  |  |  |  | Get 600MB | Pass |
|  |  |  |  |  |  |  |  | S1 Standby | Pass |
|  |  |  |  |  |  |  |  | S4 Hibernate | Pass |
|  |  |  |  |  |  |  |  | Enable/Disable | Pass |
|  |  |  |  |  |  |  |  | Hot swap | N/A |
|  |  |  |  |  |  |  |  | Uninstall | Pass |
| TLD2122 | ASUS M2N-E | AMD Athlon 64/K9 3500+ 2.20GHz | Phoenix v0802 | Nvidia 570 Ultra MCP | 1*PCI-E x16 1*PCI-E x4 2*PCI-E x1 3*PCI 2.2 | nForce 570 SLI | Vista x86 Vista x64 | Put 600MB | Pass |
|  |  |  |  |  |  |  |  | Get 600MB | Pass |
|  |  |  |  |  |  |  |  | S1 Standby | Pass |
|  |  |  |  |  |  |  |  | S4 Hibernate | Pass |
|  |  |  |  |  |  |  |  | Enable/Disable | Pass |
|  |  |  |  |  |  |  |  | Hot swap | N/A |
|  |  |  |  |  |  |  |  | Uninstall | Pass |
| TLD2055 | ASUS P4B | Intel Pentium III 1.5 GHz | Award | Intel 845 (Intel i845+ ICH2) | PCI(PCI2.2) | Intel 82801BA/BAM | XP SP3 | Put 600MB | Pass |
|  |  |  |  |  |  |  |  | Get 600MB | Pass |
|  |  |  |  |  |  |  |  | S1 Standby | Pass |
|  |  |  |  |  |  |  |  | S4 Hibernate | Pass |
|  |  |  |  |  |  |  |  | Enable/Disable | Pass |
|  |  |  |  |  |  |  |  | Hot swap | N/A |
|  |  |  |  |  |  |  |  | Uninstall | Pass |